FOG LIGHT ASSEMBLY

COMPONENTS

Fig. 242: Identifying Fog Light Assembly Components

REMOVAL

1. REMOVE ENGINE UNDER COVER NO.1 (See REMOVAL )

2. REMOVE FRONT FENDER SPLASH SHIELD SUB-ASSEMBLY (See REMOVAL )

3. REMOVE FRONT BUMPER COVER (See REMOVAL )

2008 Lexus RX 350

2008 ACCESSORIES AND EQUIPMENT Lighting - RX350

4. REMOVE FOG LIGHT ASSEMBLY LH

a. Release the 2 claw fittings, then pull out the fog light assembly LH.

DISASSEMBLY

1. REMOVE FOG LIGHT BULB

a. Remove the fog light bulb from the fog light assembly LH.

2. REMOVE FOG LIGHT UNIT LH

Fig.

Fig. 243: Identifying Fog Light Assembly LH Claws

ADJUSTMENT

1. PUT VEHICLE THESE CONDITIONS

a. Tire inflation pressure is in the specified value. (See INSPECTION )

b. Start the engine.

2. ADJUST FOG LIGHT AIM

a. The fog light aim can be adjusted by moving the aiming screw in the vertical direction.

HINT:

The optical aim moves upward when turning the screwdriver clockwise, while it moves downward

when turning the screwdriver counterclockwise.

REASSEMBLY

1. INSTALL FOG LIGHT UNIT LH

2. INSTALL FOG LIGHT BULB

Fig. 244: Adjusting Fog Light Aim

INSTALLATION

1. INSTALL FOG LIGHT ASSEMBLY LH

2. INSTALL FRONT BUMPER COVER (See INSTALLATION )

3. INSTALL FRONT FENDER SPLASH SHIELD SUB-ASSEMBLY (See INSTALLATION )

4. INSTALL ENGINE UNDER COVER NO.1

5. PUT VEHICLE THESE CONDITIONS (See ADJUSTMENT

Fig. 99: Front Fog Light - Wiring Diagram