[image: image1.jpg]



Start your cuts on the vertical and horizontal lines with the dremel and use a vise.  Continue to cut along the side of the plastic housing, adjacent to where the length of the module would be.    Leave several mm of plastic on the vertical cut for continuity and stability.  It is not visible in the photo, as the lighter assembly covers this piece.  
There is a horizontal lip that runs length wise inside the housing which needs to be shaved down to slide the module in.  
Also, there is a white plastic clip that sits atop the housing in the rear.  It keeps the lighter assembly, which is square, canted, which in turn blocks the opening for the module to fit.  Depending on how tight of a fit, you will need to shave the right angle part of this clip to cant the assembly out of the way and readjust the ground wire on the assembly to fit the module.
